__________________________________

HP 95LX Low-level Graphics Support

__________________________________

Copyright 1991 Hewlett-Packard Company.  All rights reserved.

____________

INTRODUCTION

The package of low-level routines provided in the HP 95LX that are

available for use by applications programs support these functions:

1) setting the display mode to alpha or graphics.

2) setting a fill mask.

3) getting information about the current graphics settings.

4) setting a logical origin to which all other graphics

operations are relative.

5) setting a rectangular clip region.

6) drawing an outlined, solid-filled, or pattern-filled

rectangle.

7) drawing a line.

8) plotting a point.

9) moving the current pen location.

10) setting the current pen color.

11) setting the current replacement rule.

12) setting the current line-type.

13) reading the current color value of a point.

14) reading a rectangular image.

15) writing a rectangular image.

16) labeling text using the internal 6x8 font.

All of the routines which plot to the display always obey the current

logical origin, clip region, pen color, replacement rule, and (where

appropriate) line-type and fill-mask.

The pen color can be 0 or 1.

The replacement rule can be one of FORCE, AND, OR, or XOR.  Writing

a rectangular image can optionally invert the image before applying

the specified replacement rule.  Writing an image is different from

all other plotting in that it uses an argument as the replacement

rule rather than the current replacement rule.

The line-type is a 16-bit value whose bits are used repeatedly when

drawing a line or an outlined rectangle.

The fill-mask is an 8-byte value which specifies an 8-bit by 8-bit

rectangular mask which is used repeatedly when drawing a pattern-filled

rectangle.

The general process to do graphics is:

1) set the display mode to graphics

2) set the desired pen color, replacement rule, linetype,

fillmask, logical origin, and clip region, if different

than the default values set by the set-mode function.

3) perform the desired drawing using the attributes setup by

step 2).

4) repeat steps 2) and 3) until done.

5) set the display mode back to alpha.

The graphics routines are accessed through software interrupt 5F (hex).

The required arguments are loaded into specific CPU registers, the

requested function number is loaded into the AH register, and then an

INT 5Fh instruction is executed.  Unless otherwise stated, all

functions preserve ALL registers except for AX.

X-coordinates always get larger (more positive) when moving to the

right on the display.  Y-coordinates always get larger when moving

down (towards the bottom) on the display.  The default origin is in

the top-left corner of the display.

Although the interface is designed primarily as an assembly language

interface, it is simple to write an assembly language module that can

provide a library of corresponding functions to a C program.  A sample

is provided in Appendix A.

______________

SET VIDEO MODE

Forces the current video mode to alpha or graphics and clears the

display.

Entry conditions:

AH = 0

AL = requested mode.

07h = alpha (system manager compliant)

20h = graphics (system manager compliant)

87h = alpha (non-system manager)

A0h = graphics (non-system manager)

Sub-functions 07h and 20h call the System Manager routine to change

video modes, thus letting the System Manager know that the display has

changed modes and the display contents destroyed.  This is important

for any applications which are intended to be system manager compliant.

Sub-functions 87h and A0h call the BIOS directly in order to change

display modes, thus by-passing the System Manager.  These should be

used by programs that are NOT System Manager compliant, so that they

will function correctly whether the system was booted into the System

Manager or if it was booted straight into DOS (by placing a SHELL=

command in a CONFIG.SYS file).

After a SET_MODE call to change to graphics, the defaults are:

Logical origin		(0, 0)

Clip region		(0, 0) thru (239, 127)

Pen location		(0, 0)

Pen color		1

Replacement rule	FORCE

Line-type		0FFFFh

Fill mask		0FFh,0FFh,0FFh,0FFh,0FFh,0FFh,0FFh,0FFh

Sample assembly code:

mov	ax,0020h	; set mode to GRAPHICS

int	5fh

.

.

.

mov	ax,0007h	; set mode to ALPHA

int	5fh

_____________

SET FILL MASK

Sets the eight-byte fill mask used by DRAW_RECTANGLE when pattern-

filling.

Entry conditions:

AH = 1

ES:DI = address of 8 bytes of fillmask.

The default fillmask after a mode set to graphics is eight bytes of

0ffh (which would result in a solid-fill).

The fillmask is always aligned with the byte boundaries of the display

memory, and it is then clipped by the rectangle being drawn.  This

means that as the rectangle is shifted bit-by-bit, the pattern appears

to exist on a plane behind the rectangle, and that the rectangle is

a moving window onto that plane.  It's tough to describe, and a little

experimentation should make it plain.

Sample assembly code:

fmask1	db	055h, 0aah, 055h, 0aah, 055h, 0aah, 055h, 0aah

.

.

.

assume	es:dgroup

lea	di,fmask1

mov	ah,1

int	5fh		; set fillmask pattern to FMASK1

________________________________

GET CURRENT GRAPHICS INFORMATION

Returns current information about the state of the graphics functions.

At entry:

AH = 2

ES:DI = address of a 36-byte long buffer into which the

graphics information will be placed.  At exit from

this function, the buffer contents will be:

offset  size

0   1-byte CURRENT VIDEO MODE

1   1-byte DEFAULT VIDEO MODE

2   1-word WIDTH OF DISPLAY IN PIXELS

4   1-word HEIGHT OF DISPLAY IN PIXELS

6   1-word CURRENT X-LOCATION OF PEN

8   1-word CURRENT Y-LOCATION OF PEN

10   1-word CURRENT LINE-TYPE

12   1-word CURRENT REPLACEMENT RULE

14   1-word CURRENT PEN COLOR

16   1-word CURRENT X-MINIMUM OF CLIP REGION

18   1-word CURRENT X-MAXIMUM OF CLIP REGION

20   1-word CURRENT Y-MINIMUM OF CLIP REGION

22   1-word CURRENT Y-MAXIMUM OF CLIP REGION

24   1-word CURRENT X-LOCATION OF LOGICAL ORIGIN

26   1-word CURRENT Y-LOCATION OF LOGICAL ORIGIN

28-35  8-bytes CURRENT FILL MASK (for rectangle fill)

At exit:

DX:AX = address of the 36-byte long buffer (for return to C).

Sample assembly code:

infobuf		label	byte

curmode		db	?

defmode		db	?

dspwidth	dw	?

dspheight	dw	?

curpenx		dw	?

curpeny		dw	?

curlinetype	dw	?

curreprule	dw	?

curpen		dw	?

curclipminx	dw	?

curclipmaxx	dw	?

curclipminy	dw	?

curclipmaxy	dw	?

curlogorgx	dw	?

curlogorgy	dw	?

curfmask	db	8 dup (?)

.

.

.

assume	es:dgroup

lea	di,infobuf

mov	ah,2		; read current video info into infobuf

int	5fh

__________________

SET LOGICAL ORIGIN

Sets the logical origin in terms of absolute screen pixels, regardless

of previous settings of the logical origin or clip region.  The (X,Y)

of the logical origin may be specified off of the actual physical

screen (ie, negative values or greater than (239, 127).  All other

coordinate arguments in this graphics system are relative to the

logical origin, including those used to specify the clip region.

SET LOGICAL ORIGIN resets the CLIP REGION to the entire physical

display (0,0) to (239,127).  So, if clip_region is used, it must be

set AFTER the set_logical_origin.

The default logical origin after a mode set is (0,0).

At entry:

AH = 3

CX = x coordinate

DX = y coordinate

Sample assembly code:

mov	ah,3

mov	cx,120		; move log org to approximately the

mov	dx,64		;   center of the display

int	5fh

_______________

SET CLIP REGION

This function sets the coordinates of the upper-left and lower-right

corners of the clip rectangle.

All reading/writing of the display in this graphics system is limited

(clipped) by the current CLIP REGION.

Default after mode set is (0,0) and (239,127).

At entry:

AH = 4

CX = x-minimum coordinate

DX = y-minimum coordinate

SI = x-maximum coordinate

DI = y-maximum coordinate

Sample assembly code:

mov	ah,4		; clip (limit) all drawing to the

mov	cx,120		;    right half of the display

mov	dx,0

mov	si,239

mov	dx,127

int	5fh

______________

DRAW RECTANGLE

This draws a rectangle which has two diagonally opposite corners at

the current pen location and (CX,DX).  Hence, you will usually first

do a MOVE_PEN, then a DRAW_RECTANGLE.  ALL rectangle draws obey the

current replacement rule.  The pen location is left at the starting

location (it is not changed).

If the rectangle drawn is just an outline, it is drawn using the

current line-type.  If the rectangle drawn is pattern-filled, it uses

the current fill-mask.  In all cases the current pen color and replace-

ment rule are used.

At entry:

AH = 5

AL = fill flag	0==outline, current linetype and pen color

1==solid fill, current pen color

2==pattern fill, current fillmask and pen color

CX = x-coordinate of second corner of rectangle

DX = y-coordinate of second corner of rectangle

Sample assembly code:

mov	ah,8		; move pen to (50,74)

mov	cx,50

mov	dx,74

int	5fh

mov	ah,5

mov	al,1		; solidfill rectangle to (101, 99)

mov	cx,101

mov	dx,99

int	5fh

_________

DRAW LINE

This draws a line from the current pen location to (CX,DX) using the

current pen color, linetype, and replacement rule.  The pen location

is left at the end point.  If another DRAW_LINE is executed after the

first without an intervening MOVE_PEN, the starting point is not

plotted.  This is to avoid the problem of drawing connecting lines

with a replacement rule of XOR.  Since the starting point of the second

line is the same as the ending point of the first line, it would get

plotted twice, which in XOR mode is the same as not plotting it at all.

At entry:

AH = 6

CX = x-coordinate of end point

DX = y-coordinate of end point

Sample assembly code:

mov	ah,6

mov	cx,21

mov	dx,10		; draw from current pen location

int	5fh		;   to (21, 10)

__________

PLOT POINT

This moves the current pen location to CX,DX and plots a single

point there using the current pen color and replacement rule.

At entry:

AH = 7

CX = x-coordinate of point

DX = y-coordinate of point

Sample assembly code:

mov	ah,7

mov	cx,239		; plot point at (239, 127)

mov	dx,127

int	5fh

________

MOVE PEN

This moves the current pen location to (CX,DX).  The default location

after a mode set is (0,0).

At entry:

AH = 8

CX = x-coordinate

DX = y-coordinate

Sample assembly code:

mov	ah,8

mov	cx,22		; move pen to (22, 44)

mov	dx,44

int	5fh

_____________

SET PEN COLOR

Sets the current pen color to 0 or 1.  The default after a mode

set is 1 (black).

At entry:

AH = 9

AL = new pen color (0 for white or 1 for black)

Sample assembly code:

mov	ah,9

mov	al,0		; set pen color to white (0)

int	5fh

____________________

SET REPLACEMENT RULE

The replacement rule controls how the current pen color is combined

with the existing color of a pixel on the display when performing

any plotting function (except WRITE_AREA, which has its own replacement

rule specified with each call).  If the current replacement rule is

FORCE then the resulting color of a pixel is equal to the current pen

color at the time of the plotting action.  For the other three replace-

ment rules (AND, OR, and XOR), the resulting color is the logical

operation between the current screen pixel color and the current pen

color.

The default replacement rule after a mode set is 0 (FORCE).

At entry:

AH = 10 (0Ah)

AL = new replacement rule	0==FORCE

1==AND

2==OR

3==XOR

Sample assembly code:

mov	ah,10

mov	al,3		; set replacement rule to XOR

int	5fh

_____________

SET LINE TYPE

This is a 16-bit value that is repeated over and over as each pixel

of a line or an outlined rectangle is drawn.  0xFFFF will cause solid

lines to be drawn. The default after a mode set is 0xFFFF (solid line).

At entry:

AH = 11 (0Bh)

CX = new line type

Sample assembly code:

mov	ah,11

mov	cx,0c440h	; set line type to XX000X000X000000

int	5fh

__________

READ POINT

This returns AX==the color (0 for white, 1 for black) of the requested

point.  The current pen location is not modified.

At entry:

AH = 12 (0Ch)

CX = x-location of point to read

DX = y-location of point to read

Sample assembly code:

mov	ah,12

mov	cx,49		; read point (49,57)

mov	dx,57

int	5fh

_____________________

READ AREA (get image)

This reads a rectangular area of the display into the specified

buffer.  There is an 8-byte header at the beginning (specifying

number of planes, number of bits/pixel, width of image, and

height of image.  The first two are always equal to 1 on the HP 95LX.

The size needed for the buffer is:

8 + ((x2-x1+8)/8) * (y2-y1+1) bytes

The diagonally opposite corner points (x1, y1) and (x2, y2) are

included in the read area.  Bits with a value of 0 are added to

the right end of each row of pixels (if necessary) to fill out an

integral number of bytes of data for that row.  The image is always

left justified within the buffer regardless of its byte-alignment on

the display.

At entry:

AH = 13 (0Dh)

CX = x-coordinate of corner 1

DX = y-coordinate of corner 1

SI = x-coordinate of corner 2

BP = y-coordinate of corner 2

ES:DI = address of buffer for image

Sample assembly code:

tmpbuf	db	56 dup (?)

.

.

.

assume	es:dgroup

mov	ah,13

mov	cx,24

mov	dx,55

mov	si,41		; read a rectangular area of the screen

mov	bp,70		;   from (24,55) thru (41,70) into

lea	di,tmpbuf	;   'tmpbuf'.

int	5fh

______________________

WRITE AREA (put image)

This writes to a rectangular area of the display from a specified

buffer.  The buffer should have the same eight-byte header described in

READ_AREA (above).  This call is different from all other "write"-type

calls in that it specifies its own replacement rule rather than using

the "current" replacement rule.  It expands upon the replacement

rule types by allowing the image to be inverted before being combined

in the usual fashion (according to FORCE, AND, OR, or XOR) with the

display contents.  This does not modify the contents of the buffer.

If the entire image doesn't fit on the display, none of it is drawn.

At entry:

AH = 14 (0Eh)

AL = replacement rule	0 FORCE

1 AND

2 OR

3 XOR

4 invert image and then FORCE

5 invert image and then AND

6 invert image and then OR

7 invert image and then XOR

CX = x-location of top-left corner of image destination

DX = y-location of top-left corner of image destination

ES:DI = address of image

Sample assembly code:

tmpbuf	db	56 dup (?)

.

.

.

assume	es:dgroup

mov	ah,14

mov	al,7		; invert, then XOR

mov	cx,133

mov	dx,66

lea	di,tmpbuf	; put image 'tmpbuf' at (133,66)

__________

WRITE TEXT

Writes the specified text (all 256 chars are legal EXCEPT 0)

to the specified location, horizontally or rotated 90 degrees

counter-clockwise from horizontal, using the BIOS 6x8 font, the

current pen color and replacement rule. The specified location (CX, DX)

is the top-left corner of the text string, or if the rotate flag

is non-zero, the string is rotated 90 degrees counterclockwise about

the point (CX, DX) (such that it is now the bottom-left corner of the

text).

At entry:

AH = 15 (0Fh)

AL = rotate flag.

CX = x-coordinate of top-left corner of first character

DX = y-coordinate of top-left corner of first character

ES:DI = address of null-terminated string

Sample assembly code:

txtstr	db	"This is a test",0

.

.

.

assume	es:dgroup

mov	ah,15

mov	al,0		; plot text horizontally

mov	cx,124

mov	dx,37		; at (124, 37)

lea	di,txtstr

int	5fh

___________________________________________________________________

APPENDIX A: Sample assembly language module of C-callable functions

;�������������������������������������������������������������������������

;� Graphics interface module for calling Jaguar graphics from C programs �

;� Copyright 1990 Hewlett Packard Company.  All rights reserved.	 �

;� Author: Everett Kaser  August 14, 1990.				 �

;�������������������������������������������������������������������������

.MODEL	LARGE,C

.CODE

assume	ds:nothing

;������������������������������������������������������������������������

; G_Mode(int BiosVideoMode);

;������������������������������������������������������������������������

G_Mode		PROC	BiosVideoMode:word

mov	ax,BiosVideoMode

xor	ah,ah

int	5fh

ret

G_Mode		endp

;������������������������������������������������������������������������

; G_FillMask(maskptr);

;������������������������������������������������������������������������

G_FillMask	PROC	uses ES DI, maskptr:dword

les	di,maskptr

mov	ah,1

int	5fh

ret

G_FillMask	endp

;������������������������������������������������������������������������

; G_GetInfo(G_INFO *gp);

;������������������������������������������������������������������������

G_GetInfo	PROC	uses ES DI, gp:dword

les	di,gp

mov	ah,2

int	5fh

ret

G_GetInfo	endp

;������������������������������������������������������������������������

; G_LorgA(int x, int y);

;������������������������������������������������������������������������

G_LorgA		PROC	x:word, y:word

mov	cx,x

mov	dx,y

mov	ah,3

int	5fh

ret

G_LorgA		endp

;������������������������������������������������������������������������

; G_ClipL(int xmin, int ymin, int xmax, int ymax);

;������������������������������������������������������������������������

G_ClipL		PROC	uses SI DI, xmin:word, ymin:word, xmax:word, ymax:word

mov	cx,xmin

mov	dx,ymin

mov	si,xmax

mov	di,ymax

mov	ah,4

int	5fh

ret

G_ClipL		endp

;������������������������������������������������������������������������

; G_Rect(int x, int y, int fillflag);

;������������������������������������������������������������������������

G_Rect		PROC	x:word, y:word, fill:word

mov	cx,x

mov	dx,y

mov	ax,fill

mov	ah,5

int	5fh

ret

G_Rect		endp

;������������������������������������������������������������������������

; G_Draw(int x, int y);

;������������������������������������������������������������������������

G_Draw		PROC	x:word, y:word

mov	cx,x

mov	dx,y

mov	ah,6

int	5fh

ret

G_Draw		endp

;������������������������������������������������������������������������

; G_Point(int x, int y);

;������������������������������������������������������������������������

G_Point		PROC	x:word, y:word

mov	cx,x

mov	dx,y

mov	ah,7

int	5fh

ret

G_Point		endp

;������������������������������������������������������������������������

; G_Move(int x, int y);

;������������������������������������������������������������������������

G_Move		PROC	x:word, y:word

mov	cx,x

mov	dx,y

mov	ah,8

int	5fh

ret

G_Move		endp

;������������������������������������������������������������������������

; G_ColorSel(int color);

;������������������������������������������������������������������������

G_ColorSel	PROC	color:word

mov	ax,color

mov	ah,9

int	5fh

ret

G_ColorSel	endp

;������������������������������������������������������������������������

; G_RepRule(int rrule);

;������������������������������������������������������������������������

G_RepRule	PROC	rrule:word

mov	ax,rrule

mov	ah,0ah

int	5fh

ret

G_RepRule	endp

;������������������������������������������������������������������������

; G_LineType(int ltype);

;������������������������������������������������������������������������

G_LineType	PROC	ltype:word

mov	cx,ltype

mov	ah,0bh

int	5fh

ret

G_LineType	endp

;������������������������������������������������������������������������

; G_PointRead(int x, int y);

;������������������������������������������������������������������������

G_PointRead	PROC	x:word, y:word

mov	cx,x

mov	dx,y

mov	ah,0ch

int	5fh

ret

G_PointRead	endp

;������������������������������������������������������������������������

; G_ImageGet(int x1, int y1, int x2, int y2, char far *image);

;������������������������������������������������������������������������

G_ImageGet	PROC	uses ES SI DI, x1:word, y1:word, x2:word, y2:word, image:dword

mov	cx,x1

mov	dx,y1

mov	si,x2

les	di,image

mov	bp,y2

mov	ah,0dh

int	5fh

ret

G_ImageGet	endp

;������������������������������������������������������������������������

; G_ImagePut(int x, int y, char far *image, int replacerule);

;������������������������������������������������������������������������

G_ImagePut	PROC	uses ES DI, x:word, y:word, image:dword, reprule:word

mov	cx,x

mov	dx,y

les	di,image

mov	ax,reprule

mov	ah,0eh

int	5fh

ret

G_ImagePut	endp

;������������������������������������������������������������������������

; G_Text(int x, int y, char far *string, rotflag);

;������������������������������������������������������������������������

G_Text		PROC	uses DS ES SI DI, x:word, y:word, string:dword, rotflag:word

mov	ax,rotflag

mov	cx,x

mov	dx,y

les	di,string

mov	ah,0fh

int	5fh

ret

G_Text		endp

@curseg	ends

end

______________________________________________________________________

APPENDIX B: Sample header file for use with C programs and APPENDIX A.

/*�����������������������������������������������������������������������

� Definitions.							�

�����������������������������������������������������������������������*/

#define	G_ALPHA		0x07

#define	G_GRAPHICS	0x20

#define	G_FORCE		0

#define	G_AND		1

#define	G_OR		2

#define	G_XOR		3

#define	G_NOTFORCE	4

#define	G_NOTAND	5

#define	G_NOTOR		6

#define	G_NOTXOR	7

#define	G_OUTLINE	0

#define	G_SOLIDFILL	1

#define	G_PATTERNFILL	2

#define	MINCOLOR	0

#define	MAXCOLOR	1

/*�����������������������������������������������������������������������

� Structures.								�

�����������������������������������������������������������������������*/

typedef	struct g_info {

unsigned char	vidmode;

unsigned char	defmode;

unsigned int	xpixels;

unsigned int	ypixels;

int		xloc;

int		yloc;

unsigned int	linetype;

int		rrule;

unsigned int	color;

int		xclipmin;

int		yclipmin;

int		xclipmax;

int		yclipmax;

int		xlorg;

int		ylorg;

unsigned char	fillmask[8];

} G_INFO;

/*�����������������������������������������������������������������������

� Graphics library function definitions.  All x,y locations are rel-	�

�   ative to the current logical origin unless specified otherwise.	�

�����������������������������������������������������������������������*/

void   far cdecl	G_Mode(int);

G_INFO far * cdecl	G_GetInfo(G_INFO far *);

void   far cdecl	G_ColorSel(int);

void   far cdecl	G_RepRule(unsigned int);

void   far cdecl	G_LineType(unsigned int);

void   far cdecl	G_FillMask(unsigned char far *);

void   far cdecl	G_LorgA(int, int);

void   far cdecl	G_ClipL(int, int, int, int);

void   far cdecl	G_Move(int, int);

void   far cdecl	G_Point(int, int);

int    far cdecl	G_PointRead(int, int);

void   far cdecl	G_Draw(int, int);

void   far cdecl	G_Rect(int, int, int);

void   far cdecl	G_ImageGet(int, int, int, int, char far *);

void   far cdecl	G_ImagePut(int, int, char far *, int);

void   far cdecl	G_Text(int, int, char far *, int);

/*�����������������������������������������������������������������������*/

/**** G_MODE(mode): mode = {G_TEXT | G_GRAPHICS}

changes the display mode to text or graphics		*/

/**** G_GETINFO(GraphInfoPtr): GraphInfoPtr is a far pointer to a buffer of

the programmers choosing where the graphics information will be copied.

See the typedef for the G_INFO structure in this file for the contents

of the buffer.	*/

/*�����������������������������������������������������������������������*/

/**** G_COLORSEL(color): color = 0 or 1

sets the current pen to "color"				*/

/**** G_REPRULE(reprule): reprule = {G_FORCE | G_AND | G_OR | G_XOR}

sets the current replacement rule for all other drawing (except for

G_ImagePut, which specifies its own replacement rule). */

/**** G_LINETYPE(linetype): linetype = a 16-bit image that is repeated while

drawing lines and G_OUTLINE'd rectangles.  Bits that are 1 cause the

current pen color to be plotted using the current replacement rule.

Bits that are 0 are not plotted and leave the display un-modified. */

/**** G_FILLMASK(buffer): buffer is a far pointer to an 8-byte array which

specifies the fillmask to use when doing drawings of rectangles

with a fillflag of G_PATTERNFILL.  Every bit that is a 1 will

cause a point of the current color to be plotted.  Every bit that is

a 0 will cause that "point" of the display to be undisturbed. */

/*�����������������������������������������������������������������������*/

/**** G_LORGA(x,y): x,y = -32768 to +32767

sets the logical origin (0,0) to be located at the absolute screen

coordinate specified by x,y. */

/**** G_CLIPL(x1, y1, x2, y2): x1,y1,x2,y2 = -32768 to +32767

sets the current clip boundary to the rectangle whose diagonally

opposite corners are specified by the absolute screen coordinates

equal to the x1,y1 and x2,y2 offsets from the current logical origin.*/

/*�����������������������������������������������������������������������*/

/**** G_MOVE(x, y): x,y = -32768 to +32767

causes the logical pen to be moved to "x,y"		*/

/**** G_POINT(x,y): x,y = -32768 to +32767

plots a point of the current pen color with the current replacement

rule at "x,y" unless "x,y" is outside the current clip limits	*/

/**** G_POINTREAD(x,y): x,y = -32768 to +32767

reads the color of the point located at x,y and returns that as the

value of the function. */

/**** G_DRAW(x,y): x,y = -32768 to +32767

draws a line of the current pen color with the current replacement rule

and the current linetype from the current pen location to "x,y";

only those points lying within the clip limits are actually plotted. */

/**** G_RECT(x,y,fillflag): x,y = -32768 to +32767

fillflag = {G_OUTLINE | G_SOLIDFILL | G_PATTERNFILL}

draws a rectangle with diagonally opposite corners at the current pen

location and at "x,y", using the current pen color and replacement

rule.  The "type" of rectangle drawn is determined by fillflag.

if fillflag==G_OUTLINE, the outline of a rectangle is drawn, using the

current linetype.

if fillflag==G_SOLIDFILL, a solid, filled rectangle of the current

color is drawn.

if fillflag==G_PATTERNFILL, a pattern filled rectangle of the current

color is drawn, using the current fill mask.	*/

/*�����������������������������������������������������������������������*/

/**** G_IMAGEGET(x1, y1, x2, y2, buffer):  x1,y1,x2,y2 = -32768 to +32767

If both points x1,y1 and x2,y2 are within the current clip boundary,

the display image bounded by the rectangle whose diagonally opposite

corners are x1,y1 and x2,y2 is read into the bytes pointed to by

"buffer". */

NOTE: for G_ImageGet(), the required size of 'buffer' is (on Jaguar):

8 + ((x2-x1+8)/8) * (y2-y1+1) bytes          */

/**** G_IMAGEPUT(x, y, buffer, reprule): x,y = -32768 to +32767

reprule = {G_FORCE | G_AND | G_OR | G_XOR | G_NOTFORCE | G_NOTAND |

G_NOTOR | G_NOTXOR}

if x,y and the un-specified bottom-right corner of the image are

within the clip boundary, the image from "buffer" is drawn on the

display using reprule as the replacement rule.  (For the "NOT" replace-

ment rules, the image is color-inverted first, then placed on the

display using the "rest" of the replacement rule.) */

/*�����������������������������������������������������������������������*/

/**** G_TEXT(x,y,buffer,rotflag): x,y = -32768 to +32767, rotflag = 0 or 1

draws the string pointed to by "buffer" on the display at the specified

location x,y using the current font. If rotflag = 0, it's drawn

horizontally, else it's rotated 90 degrees counterclockwise. */

/*�����������������������������������������������������������������������*/