����������������������������������������������Ŀ

� BATT100.COM: Battery Status for the HP 100LX �

������������������������������������������������

Version 1.1

BATT100 is a utility that provides a complete report

of the battery status of your HP 100LX. It can be

launched from DOS, Filer, or AppManager. BATT100 can

also log the voltage with a time and date stamp to

a file if it is renamed BATT100L.COM.

Example AppManager Installation

1. Copy BATT100.COM and BATT100.ICN to C:\

2. Press [More] and [F2]:Add

3. Enter as follows:

Name:  &Battery

Path:  c:\batt100.com

Comments:  �

4. Select icon with arrows and press [F10].

You can now launch it by highlighting its icon

and pressing [Enter] or simply pressing [B] if

it is the first B hotkey in your list.

Main Battery Level

This is the voltage of your main AA batteries at

the time the program was launched. The 100LX reports

a Low Battery condition at approximately 2.20 volts.

It is not reliable to use this voltage to determine the

remaining life for Lithiums, Nicads, or Nickel-Metal

Hydride batteries as their discharge curve is very flat until

the end of the battery's charge.

Main Battery Type

This is the setting that is reported in the SETUP

application. It will be either Alkaline or Nicad.

Charging Status

This reports the status of the recharging circuit in

the HP 100LX. It will display FAST for the first 6

hours of charging and SLOW while it is trickle

charging. It will display OFF if the charging is

off and DISABLED if Alkaline is selected.

Fast Charge Time

This is the amount of time that the 100LX has been

charging the batteries at its 80ma rate Fast charge.

This value will vary up to 6 hours at which time

it will switch to the 40ma Slow rate.

Backup Battery Level

This is the voltage level of your backup battery at

the time the program was launched. Replacement is

recommended if the voltage drops below 2.80 volts.

Card Battery Status

This is the output of the Card battery level circuit.

It will display either OK or LOW. You should replace

it as soon as possible when you get a LOW report.

Remember the card battery is in use whenever the card

is removed or in the 100LX and the unit is off and

the AC adapter is not connected.

Logging to a File

By renaming BATT100.COM to BATT100L.COM the main battery

voltage, time, date, and battery type are logged to a

file named BATT100.LOG in the C:\_DAT directory each time

the program is run. The file is a simple text file where

each line is in the following form:

[main battery voltage] [time] [date] [battery type]

BATT100.COM is copyrighted Freeware. Commercial

distribution is not permitted without express written

permission from its author. For permission or other

questions write to:

Mark V. Scardina

450 Taraval St. Ste. 302

San Francisco, CA. 94116

CIS ID: 76711,732

(c)Copyright 1993     Mark V. Scardina