CHARGE-IT!

v 1.02

CHARGE-IT! is a battery charging controller/monitor meant to provide a

more flexible and sophisticated alternative to the standard LX 6-hours

fast charge then trickle charge cycle.

Although it has some of the same functions, it's NOT a clone of ABC/LX,

or a substitute.  CHARGE-IT! doesn't have a TSR component to track

battery voltage during use.  If you want complete care-free monitoring &

control of your batteries, you should consider contacting D&A Software

about ABC/LX.  CHARGE-IT! was written mostly as a fun exercise in Forth.

What CHARGE-IT! will do is to let you set several parameters for

overnight charging (maximum voltage, time limit, and time that voltage

doesn't rise).  You can also set a voltage to enable charging

automatically when starting CHARGE-IT!.  During charging it will graph

the battery voltage.  Parameters are entered on the command line.

It's probably easiest to do this with a small batch file.  A sample

is included.  You can modify this for your own settings.  Voltage

settings are mV (ie  2.91V = 2910), and time is in minutes.  If you

want to trickle charge after fast charging is done, add the trickle

charge time (in minutes) and the word slo

to the command line. For example:

charge.com  2450 vmin  2940 vmax  600 tmax  120 stall  60 slo

will launch CHARGE-IT! and set it to enable charging at or below

2.45V, stop fast charging at 2.94V OR after 10 hours OR if voltage

doesn't rise for 2 hours, then slow charge for 1 hour.  You can enter

parameters in any order, and leave any out (there are default values

preset).  HOWEVER, THE VALUE OF THE PARAMETER MUST COME FIRST, AND

PARAMETER NAMES MUST BE IN LOWER CASE!!  This program is my first

attempt to write a (I hope) useful app in Forth.  The parameter names

are actually Forth words that load program variables with their

values.  The program has a little intelligence, and limits parameters

to reasonable values.

The preset defaults are:

Enable charge at:   2.4 V

Stop charging at:   3.0 V     OR

after:   8 Hours   OR if

stalled for:   90 Minutes

Final slow charge:   off

You can start CHARGE-IT! from a DOS prompt or from an icon in AppMgr

X-Finder, HDM, or SMMx.  An icon is included.  If you run it under

SysMgr, it has a LIMITED ability to task-swap.  It can't control

charging unless it's active, so you MUST return to CHARGE-IT! to

control charging, but the timers should still be correct on return,

and the graph will continue from where it left off.  The program needs

a minimum of 64K to run.

When it's fired up, you'll see the graph screen (Unless your battery

type is set for Alkaline - then you'll get a warning).  It shows the

battery voltage & current charging status (Charging Disabled, Not

Charging, Fast Charging, or Slow Charging).  If the batteries are at

or below the start voltage, charging will be enabled ("Not Charging"

if the adapter isn't hooked up - or - "Fast Charging" if it is).  The

graph is only drawn during charging.  Four of the function keys

control charging manually:

F3 (Start) turns on charging (fast).

F5 (Slow) turns on slow charging, or toggles slow or fast charging

(the label will change).

F7 (Stop) stops charging.

F10 (Exit) stops charging and exits the program. (ESC does the same

thing).

Slow charging can only be selected with the adaptor connected.  If you

choose "Slow" without the adaptor, nothing happens...

At the conclusion of fast charging, the F1 key label will change to a

white box with the reason fast charging stopped.  (Tmax, Vmax, or

Stall).

When you exit with F10 or ESC, there'll be a brief message, and a list

of control-variable values, and the limits.  This was originally for

trouble-shooting, but what the hell??

What's here:

charg-it.doc   This file

charge.com     The executable

charg-it.bat   A sample batch file to set parameters

charg-it.icn   The icon

charge.scr     Forth source code

compile.doc    Very brief instructions for recompiling

This release of Charge-It! now includes the Forth source code blocks.

If you try to read it with most file viewers, it'll look strange to

say the least.  Pygmy source code contains no CR/LF characters to

separate the lines!  It can be viewed with QuickView, but it's only

useful with HP-Pygmy Forth (Release 2), since it uses many Palmtop-

specific extensions.  Incidentally, HP-Pygmy was accidentally released

with the code for compiling Charge-It! - on block 175.  See

COMPILE,DOC for more.

The changes in this version are mostly cosmetic.  The battery error

message looks nicer if it pops up, and the decision to use the 12 or

24 hour graph screens is now based on the total time limit of fast and

slow charging, not just fast.

CREDITS:

This program is the result of an idea from Jim Dantin, who provided

suggestions for what should be included, several improvements during

development, and helped with the internal logic. (And also took the

risk of testing early versions...)  Thanks also to Jorgen Dybdahl for

letting me in on the secret of turning on slow charging, and defeating

the 6-hour charge time limit.  And of course many thanks to Frank

Sergeant, the author of Pygmy Forth.

FEE:

There ain't no fee!  This program is freeware.  But if you feel some

strange, inexplicable urge to send me money, I won't say no...   8-)

Peniel Romanelli

peniel@web2000.net

19 August 1999